# Pose2Sim vs RajagopalLaiUhlrich2023 (RLU2023) kinematic models

Kinematic models were parsed directly from the model files (.osim), not taken from documentation.

- Files: Model_Pose2Sim_simple.osim (Pose2Sim), RajagopalLaiUhlrich2023.osim (OpenSim), Markers_Halpe26.xml (Pose2Sim).

- Ranges are in degrees except pelvis translations (m).

- Clamped = OpenSim enforces the range during IK. Locked = the DOF is fixed at its default value (0 for all locked coordinates here).

- Segment coordinate system axes in the neutral pose: +x anterior, +y superior, +z lateral.

## Pose2Sim model

22 bodies, 22 joints and 40 DoF.

### Segments

| Body | Parent joint | Parent body |
| --- | --- | --- |
| pelvis | ground_pelvis | ground |
| sacrum | sacrumjnt (weld) | pelvis |
| femur_r/l | hip_r/l | pelvis |
| tibia_r/l | walker_knee_r/l | femur_r/l |
| talus_r/l | ankle_r/l | tibia_r/l |
| calcn_r/l | subtalar_r/l | talus_r/l |
| toes_r/l | mtp_r/l | calcn_r/l |
| torso | back | pelvis |
| head | head_torso | torso |
| humerus_r/l | acromial_r/l | torso |
| ulna_r/l | elbow_r/l | humerus_r/l |
| radius_r/l | radioulnar_r/l | ulna_r/l |
| hand_r/l | radius_hand_r/l | radius_r/l |

### Joints, DOF and coordinate ranges

| Joint | Type | DOF | Coordinates and ranges | Clamped | Locked |
| --- | --- | --- | --- | --- | --- |
| ground_pelvis | CustomJoint | 6 | pelvis_tilt, pelvis_list, pelvis_rotation −90 to 90; pelvis_tx −5 to 5 m, pelvis_ty −1 to 2 m, pelvis_tz −3 to 3 m | No | No |
| sacrumjnt | WeldJoint | 0 | — | — | — |
| hip_r/l | CustomJoint | 3 | hip_flexion −50 to 150; hip_adduction −60 to 30; hip_rotation −70 to 40 | Yes | No |
| walker_knee_r/l | CustomJoint | 1 | knee_angle 0 to 155 (secondary rotations and translations coupled to it by splines) | Yes | No |
| ankle_r/l | CustomJoint (one oblique axis) | 1 | ankle_angle −60 to 60 | Yes | No |
| subtalar_r/l | PinJoint | 1 | subtalar_angle −20 to 20 | Yes | No |
| mtp_r/l | PinJoint | 1 | mtp_angle −30 to 30 | Yes | No |
| back | CustomJoint | 3 | L5_S1_Flex_Ext, L5_S1_Lat_Bending, L5_S1_axial_rotation −90 to 90 | Yes | No |
| head_torso | CustomJoint | 3 | neck_flexion −29 to 29; neck_bending −90 to 90; neck_rotation −45 to 45 | Yes | No |
| acromial_r/l | CustomJoint | 3 | arm_flex −90 to 90; arm_add −120 to 90; arm_rot −90 to 90 | No | No |
| elbow_r/l | PinJoint | 1 | elbow_flex 0 to 150 | Yes | No |
| radioulnar_r/l | PinJoint | 1 | pro_sup 0 to 120 | Yes | No |
| radius_hand_r/l | UniversalJoint | 2 | wrist_flex −110 to 110; wrist_dev −25 to 35 | Yes | No |

DOF total is 6 + 2 × 7 (legs) + 3 (lumbar) + 3 (neck) + 2 × 7 (arms) = 40.

## RajagopalLaiUhlrich2023 model

22 bodies, 22 joints and 37 DOF, with 8 locked by default, so 29 free DOF. Patella are driven from knee angle.

Locked DOF are: locking the 3-segment foot into a 1-segment (-2 DOF per side: subtalar and MTP), and locking wrist (-2 DOF per side: wrist FE and radio ulnar deviation)

### Segments

| Body | Parent joint | Parent body |
| --- | --- | --- |
| pelvis | ground_pelvis | ground |
| femur_r/l | hip_r/l | pelvis |
| tibia_r/l | walker_knee_r/l | femur_r/l |
| patella_r/l | patellofemoral_r/l | femur_r/l |
| talus_r/l | ankle_r/l | tibia_r/l |
| calcn_r/l | subtalar_r/l | talus_r/l |
| toes_r/l | mtp_r/l | calcn_r/l |
| torso (includes head) | back | pelvis |
| humerus_r/l | acromial_r/l | torso |
| ulna_r/l | elbow_r/l | humerus_r/l |
| radius_r/l | radioulnar_r/l | ulna_r/l |
| hand_r/l | radius_hand_r/l | radius_r/l |

### Joints, DOF and coordinate ranges

| Joint | Type | DOF | Coordinates and ranges | Clamped | Locked |
| --- | --- | --- | --- | --- | --- |
| ground_pelvis | CustomJoint | 6 | pelvis_tilt, pelvis_list, pelvis_rotation −90 to 90; pelvis_tx −5 to 5 m, pelvis_ty −1 to 2 m, pelvis_tz −3 to 3 m | Yes | No |
| hip_r/l | CustomJoint | 3 | hip_flexion −30 to 120; hip_adduction −50 to 30; hip_rotation −40 to 40 | Yes | No |
| walker_knee_r/l | CustomJoint | 1 | knee_angle 0 to 140 (secondary rotations and translations coupled to it by polynomials) | Yes | No |
| patellofemoral_r/l | CustomJoint | 0 independent | knee_angle_beta, dependent on knee_angle through the coupler constraint | — | — |
| ankle_r/l | PinJoint | 1 | ankle_angle −50 to 50 | Yes | No |
| subtalar_r/l | PinJoint | 1 | subtalar_angle −35 to 35 | Yes | Yes |
| mtp_r/l | PinJoint | 1 | mtp_angle −45 to 30 | Yes | Yes |
| back | CustomJoint | 3 | lumbar_extension, lumbar_bending, lumbar_rotation −90 to 90 | Yes | No |
| acromial_r/l | CustomJoint | 3 | arm_flex −90 to 90; arm_add −120 to 90; arm_rot −90 to 90 | Yes | No |
| elbow_r/l | PinJoint | 1 | elbow_flex 0 to 150 | Yes | No |
| radioulnar_r/l | PinJoint | 1 | pro_sup 0 to 120 | Yes | No |
| radius_hand_r/l | UniversalJoint | 2 | wrist_flex −70 to 70; wrist_dev −25 to 35 | Yes | Yes |

The independent DOF total is 6 + 2 × 7 (legs) + 3 (lumbar) + 2 × 7 (arms) = 37.

## Differences between the two models

Anything not listed below is identical. Note a lot of the below are of very little consequence, only listed for completeness. I have highlighted the ones that matter.

| Item | Pose2Sim | RLU2023 |
| --- | --- | --- |
| Patella bodies and patellofemoral coupler constraints | Absent | Present (2 bodies, 2 constraints) |
| Head | Separate body (4.28 kg), torso 22.55 kg | Part of torso (26.83 kg) |
| Neck joint | head_torso, 3 DOF | None |
| Sacrum | Massless body welded to pelvis | None |
| Lumbar coordinate names | L5_S1_Flex_Ext, _Lat_Bending, _axial_rotation | lumbar_extension, _bending, _rotation (same axes) |
| Hip joint centre in pelvis (mm, R side) | −70.7, −66.1, 83.5 | −56.3, −78.5, 77.3 (P2S is 14 posterior, 12 superior, 6 lateral) |
| Knee joint centre in femur (mm, R side) | −8.1, −408.0, −2.8 | −4.5, −409.6, −1.8 |
| Ankle joint centre in tibia (mm) | 0, −430, 0 | −10, −400, 0 (P2S is 30 inferior, 10 posterior) |
| Knee coupling functions | SimmSplines defined 0–120°, flat beyond | Polynomials; coupled rotations agree within 0.1° over 0–120° |
| Ankle joint type | CustomJoint | PinJoint (same axis orientation) |
| hip_flexion | −50 to 150 | −30 to 120 |
| hip_adduction | −60 to 30 | −50 to 30 |
| hip_rotation | −70 to 40 | −40 to 40 |
| knee_angle | 0 to 155 | 0 to 140 |
| ankle_angle | −60 to 60 | −50 to 50 |
| subtalar_angle | −20 to 20, unlocked | −35 to 35, locked |
| mtp_angle | −30 to 30, unlocked | −45 to 30, locked |
| wrist_flex | −110 to 110, unlocked | −70 to 70, locked |
| wrist_dev | −25 to 35, unlocked | −25 to 35, locked |
| Clamping of pelvis and shoulder coordinates | Not clamped | Clamped |
| Markers in model file | 0 | 66 |

## Halpe26 keypoint positions

This is the way the keypoints are mapped to the model’s segments (same idea as mapping Vicon markers onto the model).

Read the Markers_Halpe26.xml file from Pose2Sim and extracted local keypoints coordinates in the table below.

Offsets are marker minus joint centre, in the marker's parent segment CS, with “joint centre” taken as “origin of parent segment CS”. The RLU2023 columns apply when the same bone-fixed markers are used on RLU2023. The pelvis, femur, tibia, torso, humerus and radius CS are identical in both models, so only the hip, knee and ankle offsets change between Pose2sSim and RJU2023 models.

Main findings:

- Hip keypoint sits ~30 mm lateral to the hip centres (femur CS origin)

- Knee keypoint sits about 12 mm proximal to the “knee joint centre” (origin of tibia CS)

- Ankles about 10mm anterior / 8mm distal / 15mm lateral

- Shoulders 30mm lateral

Note: the process to get these positions are documented in Pose2Sim part 2 paper (https://www.mdpi.com/1424-8220/22/7/2712) but that was done for the openPose keypoints (BODY_25B, not halpe26).

| Keypoint | Parent body | Offset from Pose2Sim joint centre, x / y / z (mm) | Distance (mm) | Offset from RLU2023 joint centre, x / y / z (mm) | Distance (mm) |
| --- | --- | --- | --- | --- | --- |
| RHip, LHip | pelvis | 6.8 / −15.2 / 21.9 | 27.5 | −7.7 / −2.9 / 28.1 | 29.3 |
| Hip (mid) | pelvis | 6.8 / −15.2 / 0 (from midpoint of hip centres) | 16.7 | −7.7 / −2.9 / 0 | 8.2 |
| RKnee, LKnee | femur | 2.7 / 10.1 / 3.4 | 11.0 | −0.9 / 11.7 / 2.4 | 12.0 |
| RAnkle, LAnkle | tibia | −0.3 / 21.9 / 15.0 | 26.6 | 9.7 / −8.0 / 15.0 | 19.6 |
| RShoulder, LShoulder | torso | −15.9 / −5.2 / 31.6 | 35.7 | same | 35.7 |
| RElbow, LElbow | humerus | 11.9 / −11.7 / 0.9 | 16.7 | same | 16.7 |
| RWrist, LWrist | radius | 8.6 / 0.7 / −3.9 | 9.5 | same | 9.5 |
| Neck | torso | −0.8 / −94.6 / 0.2 (from head_torso) | 94.6 | no neck joint | — |